Annual REPORT

oа the Chairperson of TC 1.4 "Flow Measurement"

from 01.01.2018 to 01.03.2019

- Meeting TC

From 01.01.2018 to 01.03.2019 г. 1 meeting was held: September 5, 2018. The meeting was held on the basis of the Federal state unitary enterprise "All-Russian research Institute of flowmetering (FGUP "VNIIR", Kazan).

- PROGRESS WITH COOMET PROJECTS

Works were realized on the below projects from 01.01.2018 to 01.03.2019:

- Project 680/RU-a/16 Comparisons of national standards for gas flow and volume in the flow range from 20 to 6500 m3/h.

The purpose of comparisons is to establish the degree of equivalence of national standards and asessment of calibration and measurement capabilities of laboratories of national metrological institutes in fluid flow.

| Partners: | VNIIR, BelGIM, PTB |
| --- | --- |
| Description: | Flow range: from 20 to 6500 m3/h. Conditions of comparisons: - measured medium-air; - test pressure, kPa-close to atmospheric; - temperature of the measured medium, °C-from 15 to 25; - outside relative humidity , % - from 30 to 80. As means of comparison are used: Turbine gas meter TRZ, size G4000, nominal diameter DN300, flow range from 320 to 6500 m3/h Gas meter rotary IRM-a-DUO, size G250, nominal diameter DN100, flow range from 20 to 400 m3/h. |
| Pilot comparisons | FGUP "VNIIR" (Russia) |

According to this project, the preparation for research and the research of the metrological characteristics of the reference standard were carried out. A technical protocol was prepared and sent for approval with the participants of comparisons.

- Project 684/MD/16 Comparison of gas consumption

The aim of the project is to establish the degree of equivalence of national standards and assessment of calibration and measurement capabilities of laboratories of national metrological institutes in the field of gas consumption.

| Partners: | National Institute of Metrology (NIM, Moldova) State Enterprise "Ivano-Frankivsk Scientific and Production Center for Standardization, Metrology and certification "(Ukraine) |
| --- | --- |
| Description: | Flow range: 0,016 to 50,0 m3/h |

According to the data of the National Institute of Metrology of Moldova at 01.12.2018 comparisons are at the stage of processing the results.

- Project 652/UA-a/14 Bilateral comparisons of low pressure gas volume and volume flow standards

The aim of the project is to held supplementary comparisons of SE "Ukrmetrteststandart" gas volume and volume flow secondary standard with PTB gas volume and volume flow standards.

| Partners: | SE "Ukrmetrteststandart" (Kiev, Ukraine), РТВ (Germany) |
| --- | --- |
| Description: | Flow range of (0,01 - 65) m3/h. Measuring control volume is (0,01 - 0,2) m3 |

- Project 406/UA-a/07 Comparison of national standards for mass and volume flow units (started in July 2008).

| Partners: | PTB (Germany), BelGIM (Republic of Belarus), VNIIR (Russia), LEI (Lithuania), NSC "Institute of Metrology" (Ukraine), SMU (Slovakia), CNE RUz (Uzbekistan). |
| --- | --- |
| Description: | Flow range: - mass flow rate from 102 to 105 kg/h; - volume flow rate from 0.1 to 102 m3/h. - measured medium - water |
| Pilot comparisons | PTB |

The main stage of comparisons was carried out in 2011: additional comparisons of national standards of liquid consumption units COOMET.M.FF-S2 were carried out. According to the results of comparisons in October 2012, the pilot organization prepared the final draft (Draft B). The results of all participants can be considered positive. In 2018, the draft (Draft B) was revised taking into account the comments of the WGFF (Working Group for Fluid Flow). No comments were received from the participants of the comparisons. The publication of the final draft  is preparing (Final Draft).

In august 2018 the Project 760/RU/18 Regional key comparisons of national standards in the field of water flow and volume in the flow range from 0,1 to 45 t/h was included in in COOMET program of comparisons.

The aim of the project is to establish the degree of equivalence of national standards and assessment the calibration and measurement capabilities of laboratories of national metrology institutes in fluid flow.

| Partners: | VNIIR (Russia), BelGIM (Republic of Belarus), KazInMetr (Kazakhstan), NSC “Institute of Metrology” (Ukraine), NIM (Moldova), CMI (Czech Republic) |
| --- | --- |
| Description: | Flow range: from 0.1 to 45 t/h. Terms of comparisons: - measured medium - water; - temperature of the measured medium, ° С - from 15 to 25. Mass flow meters Promass are used as the means of comparison. |
| Pilot comparisons | FGUP "VNIIR" (Russia) |

According to this project, preparation for research and research of metrological characteristics of the secondary transfer standard were carried out.

At the moment, the technical protocol is being finalized on the comments received from the participants in the comparisons.

Updating COOMET program of comparisons was realized in February 2019 and 3 project were finished:

- Project 679/RU/16 Comparisons of national standards for gas flow and volume in the flow range from 1 to 100 m3/h.

The aim of the project was to establish the equivalence of standards, transmitted gas flow units. Critical nozzles were used as vector standards.

| Partners: | VNIIR (Russia), National Institute of Metrology (NIM) China, PTB |
| --- | --- |
| Description: | Comparison conditions: - measured medium - air; - ambient temperature, ° C - from 15 to 25; - outside relative humidity % - from 45 to 80; - atmospheric pressure, kPa - from 86 to 106. |

The results of these trilateral comparisons were published at the International Metrology Conference FLOMECO, Australia ("The comparison of low pressure gas flowrate among NIM, PTB and VNIIR" / Chunhui Li, Bodo Mickan, Ilya Isaev // FLOMECO 2016, Sydney, Australia, September 26- 29, 2016).

In May 2017, after passing an interregional examination, FGUP “VNIIR” received SMS lines in the agreed range of comparisons.

- Project 452/SK-a/09 Comparison of cold water flow calibration laboratories in a range of flow rates (3 - 20) m3/h.

The aim of the project was to compare the performance of water flow calibration laboratories.

| Partners: | SMU (Slovakia), INIMET (Cuba), CENAM (Mexico), LEI (Lithuania), GP IFCSMC Ivano-Frankovsk (Ukraine) |
| --- | --- |
| Description: | Reference conditions: - the calibration medium will be water with el. conductivity ≥ 200 μS/cm - water temperature: (20 ± 5) °C -- ambient temperature range: 15 C to 25 °C - ambient relative humidity range: 45 % to 75 % - ambient atmospheric pressure range: 86 kPa to 106 kPa (0,86 bar to 1,06 bar) Meters used for the comparison will be two flowmeters KROHNE Optiflux 2010 C/D DN 40 and Meistream Plus 40. Characteristics are tested in the flow range (3-20) m3/h. |

- Project 412/UA/07 Realization of reference gas flow meter's test rigs comparisons in flow range: (4-160) m3/h.

| Partners: | VNIIR (Russia), SMU (Slovakia), LEI (Lithuania) |
| --- | --- |
| Описание: | Testing conditions: Gas: air Pressure: close to the atmospheric pressure Temperature : 18 OC to 22 OC Reference gas flow meter's test rigs comparisons were realized by means of reference gas meter with three-blade impellers Delta S-Flow G100. |

Project 585/UA-a/12"Supplementary comparisons of critical nozzle - standards of transfer units for gas flow" was excluded from COOMET program of comparisons due to the lack of interested participants of comparisons on the basis of the decision of the 12th meeting of TC 1.4, 2015.

According to the project 685/MD/16 Comparisons of liquid flow rate standards from the pilot comparisons of the National Institute of Metrology (NIM, Moldova) received a statement on the closure of the topic as irrelevant. The proposal to close this topic will be included in the agenda of the next meeting of TC 1.4.

- COOPERATION WITH INTERNATIONAL AND REGIONAL ORGANIZATIONS

In 2016 – 2018 comparisons were made within the framework of EURAMET 1396 Bilateral inter-comparison in the gas flow range from 1 m3/h to 250 m3/h.

The aim of the comparisons was to compare the metrological characteristics of the standards in the calibration of critical nozzles in the laboratories, reproducing the unit of gas flow, where as a reference standard used "bell measuring".

| Partners: | PTB (Germany), CMI (Czech Republic), FGUP "VNIIR" (Russia) |
| --- | --- |
| Description: | Flow range: 1 to 250 m3/h. The medium is air at atmospheric pressure. |
| Pilot comparisons | CMI comparisons (Czech Republic) |

The comparison draft was published on 2 November 2018. A intraregional expertise was completed and a set of documents was sent for passing the inter-regional BIPM examination for changing СМС linesVNIIR-13.04.

- PARTICIPATION IN IMPLEMENTATION OF AGREEMENT ON MUTUAL RECOGNITION OF NATIONAL MEASUREMENT STANDARDS

In the framework of cooperation with international and regional organizations, members of TC 1.4 "Flow measurement", carried out the examination of CMC lines submitted by regional metrological organizations. From 01.01.2018 to 01.03.2019 3 examinations were carried out, submitted:

- Asia Pacific Metrology Programme (APMP) projects APMP.M.46.2018, APMP.M.45.2018;

- European Association of Metrology Institutes (EURAMET) project EURAMET.M.57.2018.

- other business

In preparation for voting on the extension of the powers of V. A. Fafurin as the Сhairman of TC 1.4, in January 2019, the membership of TC 1.4 “Flow measurement” was updated. The updated embership of TC 1.4 was sent to the COOMET Secretariat and posted on the website www.coomet.net.

Based on the decision of the 23rd Council meeting of the President of COOMET in the period from 15.01.2009 to 10.02.2009 was held absent voting among the members of TC 1.4 to extend the powers of Fafurin Viktor Andreevich as Chairman of TC 1.4 "Flowmetry".

The results of the voting (attached) were sent to the COOMET Secretariat, the Chairman of the COOMET MSJC, to the COOMET national Secretariat in Russia.

Victor Fafurin

Chairperson TC 1.4 “Flow measurement”

Voting results regarding prolongation of terms of office of TC 1.4 “Flow measurement” Chairperson (Victor Fafurin, FGUP “VNIIR”, Russian Federation)

|  | Член ТК 1.4 / TC 1.4 Member | Позиция / position |
| --- | --- | --- |
| AZ | Мовлуд Гаралов / Movlud Garalov | Согласен / Agree |
| AM | Карапет Саргсян / Karapet Sargsyan | Согласен / Agree |
| BY | Александр Бардонов / Aleksandr Bardonov | Согласен / Agree |
| BG |  | Ответ не получен / no response |
| DE | Бодо Микан / Bodo Mikan | Согласен / Agree |
| GE | Сосо Рогава / Soso Rogava | Согласен / Agree |
| KZ | Алексей Матвеев / Aleksey Matveev | Согласен / Agree |
| KG | Марина Денисова / Marina Denisova | Согласен / Agree |
| CN | Ванг Чи / Wang Chi | Ответ не получен / no response |
| KP | Чое Йонг Чол / Choe Yong Chol | Ответ не получен / no response |
| CU | Жозе Франко / Jose I.Franco | Ответ не получен / no response |
| LT | Гедиминас Зигмантас / Gediminas Zigmantas | Согласен / Agree |
| MD | Виктор Грушка / Victor Grushka | Согласен / Agree |
| RU | Виктор Фафурин / Victor Fafurin | Согласен / Agree |
| RO | А.Онческу / A. Oncescu | Ответ не получен / no response |
| SK | Вильям Мазур / William Mazur | Согласен / Agree |
| TJ | Бахтиер Ашуров / Bahtier Ashurov | Согласен / Agree |
| UZ | Фуркат Курбонов / Furkat Kubonov | Согласен / Agree |
| UA | Геннадий Народницкий / Gennady Narodnitsky | Согласен / Agree |