ANNUAL REPORT

TC 1.9 “Ionizing Radiation and Radioactivity”

for 2018

1. TECHNICAL committee MEETING

The 15th meeting of TC 1.9 “Ionizing radiation and radioactivity” was held on September 18-20, 2018 in St. Petersburg, Russia.

Representatives of Belarus (BelGIM), Germany (RTV), Cuba (CENTIS, CPHR), Kazakhstan (AF National Association of National Examinations), Moldova (INM), Russia (VNIIM, VNIIFTRI), Ukraine (NIM) were present.

Contemporaneously with the TC meeting, the TC members participated in the scientific-practical conference "the VNIIM ionizing radiation department 100th anniversary".

THE MAIN TOPICS

1.1 Information on the COOMET projects in the thematic field "Ionizing radiation and radioactivity"  implementation is presented in Section 2 of the report.

1.2  Information on the International Metrology Organizations Activity

Information is presented on the COOMET President report content, on  the COOMET Secretariat Report, on the  COOMET activities in the field of the MRA implementation, legal metrology, the COOMET Quality Forum activities, the COOMET Technical Committees activities, new COOMET publications, changes in the COMET management and structural bodies, on conferring the honorary title “Honored Metrologist of COOMET”.

Report on the regular meetings of the Consultative Committee on Ionizing Radiation BIPM (CCRI) and its sections (CCRI(I) – dosimetry of x-ray and gamma radiation and charged particles, CCRI(II) – measurement of radionuclide activity, CCRI(III) – neutron measurement).

The Information is presented on the CCRI and its sections structure and objectives, issues discussed at the sections sessions, the CCRI plans for the near future activities. Particular attention is paid to issues related to the new edition of the KCDB 2.0 key comparisons database and future changes in the CMC views and submits, as well as the introduction of the ICRU 90 report “Key data for ionizing radiation dosimetry: measurement standards and applications.”

Following the discussion, it was decided to support EURAMET proposals for CMC optimization.

1.3 Organizational issues

Decisions are maked:

1. Extend the authority of the TC Chairman for the next four-year term.

2. To release I. Kharitonov from his duties as coordinator of radioactivity section and the expert on radioactivity. To entrust the duties of the coordinator of the TC activities in radioactivity measurements to I. Alekseev (VNIIM).

3. Include in the group of TC experts:

I. Alekseev – the expert in radioactivity measurements;

A. Didyk – the expert in neutron measurements.

2. THE present status OF THE COOMET PROJECTS  IN THE field of IONIZING RADIATION AND RADIOACTIVITY

2.1. Review of ongoing and completed topics and information on the results.

135/RU/99 Status of the measurement standards in the field of ionizing radiation and radioactivity in the countries-members of the COOMET

Permanent project. The aim of this work is to create a COOMET database on standards in the field of ionizing radiation and radioactivity. Participants - representatives of all COOMET member countries.

Within the framework of the implementation of the theme, representatives of COOMET member countries regularly provide information on the state of the reference base and the ongoing changes:

- list of national and other standards;

- metrological and technical characteristics of standards;

- information on conducted and planned comparisons;

- information about the work on the improvement of standards .

Participants of the meeting (L. Buyermann (PTB), V. Evseev (NSC “IM”), 
S. Soroka (BelGIM), P. Oropesa (CENTIS), G. Salas (CPHR), E. Lukyan (INM), N. Mamyrbek (AF NACE JSC), S. Biryukov (VNIIFTRI)) described the reference measurement tools available at their NMI and the measurement capabilities of ionizing radiation, changes in the composition of equipment and laboratory personnel.

A letter with E. Shakhverdiyev  report on the state of affairs in the Azstandart Committee (Azerbaijan) was read out.

635/DE/14 Comparison of the national standards of air kerma for x-rays qualities used in radiation protection, general diagnostic radiology, computed tomography and mammography COOMET.RI(I)-S4 (bilateral PTB and CHPR, pilot – PTB, Germany)

The aim of this comparisons is to support CPHR CMC entries for air kerma, air  kerma-length products for selected X-ray qualities used in radiation protection, general diagnostic radiology, computed tomography and mammography.

Three spherical cameras for protective levels (type TM32002), two cameras for mammography (such as Radcal 10x5-6M), two CT cameras (types TM77336 and TW30009), and one camera dose per area (type TA34028 ) were used.

Comparisons are completed with positive results. Considered Draft B is sent to the BIPM for placing in the database.

641/BY/14 Comparison of the national standards of air kerma for x-rays qualities used in radiation protection and diagnostic radiology COOMET.RI(I)-S3 (pilot – BelGIM, Belarus).

Supplementary comparison of the national standards of air kerma for ISO 4037 narrow spectrum series (ISO 4037-1) and IEC 61267 RQR series (IEC 61267). The aim of the comparison is to publish new and/or support existing CMC entries for air kerma for the selected x-rays qualities used in radiation protection and diagnostic radiology.

The comparison procedure consists in calibrating three ionization chambers in terms of air kerma   usingX-ray radiation modes N-40, N-60, N-80, N-100, N-120, N-150, N-200, N-250 , RQR3, RQR5, RQR7, RQR9, RQR10. It is preferable to calibrate the cameras in all selected modes. It is allowed to calibrate chambers on five modes with a narrow spectrum and on three modes of the RQR series at a minimum. If a participant does not have RQR series modes in his laboratory, he can take part in comparisons only on modes with a narrow spectrum.

Comparison participants: VNIIM (Russia), BelGIM (Belarus), PTB (Germany), GEOSTM (Georgia), CPHR (Cuba), NIM (Moldova), Azstandart (Azerbaijan), IAEA.

At present all the declared participants have completed the measurements, but the coordinator received an appeal from three more laboratories about including them in the participants list - Ukraine, Kazakhstan and Kenya. It was decided to expand the list of participants and continue measurementsaccording the program of comparisons. Final results should be presented at the next TC meeting.

410/UА/07 COOMET regional comparison of the national standards of absorbed dose to water for Co-60 gamma-radiation (pilot – VNIIFTRI, Russia)

The aim of the project is to provide traceability of absorbed dose to water measurements in the COOMET national laboratories to the BIMP key comparison reference value.

Participants expected: VNIIFTRI (Russia), CPHR (Cuba), BelGIM (Belarus), SMU (Slovakia), PTB (Germany) and the IAEA.

When the comparison was announced, the Kharkov Metrology Institute was planned to be the coordinator, in 2015 the FSUE “VNIIFTRI” assumed the role of coordinator.

Equipment for comparisons by the VNIIFTRI: national standard for the Co-60  gamma radiation absorbed dose and the absorbed dose rate, participated in the BIMP key comparisons.

It was decided to entrust the comparison coordinator A. Berland (FSUE VNIIFTRI) to send out a questionnaire to the potential participants before the end of 2018; based on the survey results, prepare a draft Technical Protocol and send it to participants before 01.03.2019.

Estimated start date of comparisons: 2019/

389/RU/07 Comparison of re-entrant ionization chambers (“dosecalibrators”) calibration factors for medical radionuclides (pilot – VNIIM, Russia)

The aim of the comparison is to ensure the traceability of the technetium-99m measurements in the national metrology institutes and laboratories of the COOMET to the BIPM.RI (II)-K4.Tc-99m and BIPM.RI (II)-K4.F-18 key comparisons reference values (through the VNIIM results).

The first cycle of comparisons was carried out in 2017.

The work was stalled due to problems with the transportation of comparison instrument by participants from Cuba. The main reason for the delay is problems with logistics.

It was decided to instruct the comparison coordinator (I. Alekseev, VNIIM) to consider two possible solutions to the problem:

- assess the possibility of transportation by water transport;

- estimate the prospect, possible terms and cost of creating a transportable analogue of a low-pressure gas device with a safety data sheet for transportation by air.

In addition, it is necessary to further discuss the list of radionuclides, in the measurement of which participants are most interested, and, if necessary, expand the program of comparisons.

2.2. Proposal of new comparisons

1 The VNIIFTRI representatives suggested to establish a new comparison in the field of radon volumetric activity. VNIIFTRI has the necessary equipment and experienced qualified personnel for carrying out comparisons. It was decided to instruct VNIIFTRI (S. Biryukov) to hold consultations with potential participants of comparisons by the end of 2018, evaluate the prospects for such comparisons, and, in case of a positive result, develop and send a draft Technical Protocol to potential participants.

2 BelGIM representatives suggested starting a new cycle of comparisons on radionuclide solutions specific activity measuring due to the ten-year period of legitimacy of previous comparisons is expirated.

After discussion, it was decided to us efor the future comparison the Barium-133 solution. The organization and conduct of comparisons should be entrusted to VNIIM (I. Alekseev).

It is decided: I. Alekseev should to evaluate the possible timeline, procedure and number of participants, to develop and distribute to participants a draft Technical Protocol.

3 The NSC “IM” (Ukraine) representative V.Yevseyev proposed to organize comparisons to measure the natural radionuclides activity in bulk sources. The relevance of comparisons is beyond doubt. It is necessary to decide on the choice of medium and type of sources, the list of participants and the pilot laboratory.

It was decided to create a working group, which should determined the technical issues:

- selection of radionuclides (radionuclides) for comparisons;

- selection of the source design for comparisons (form, volume, filler material, etc.);

- method for determining the reference value.

3. Activities in field of IMPLEMENTATION OF MUTUAL RECOGNITION OF NATIONAL MEASUREMENT STANDARDS

The SMS reviewing current state

The following CMCs were reviewed

EURAMET.RI.29.2017  – Croatia

SIM.RI.15.2018 – Canada

SIM.RI.16.2018 – Mexico

APMP.RI.11.2019 – Thailand

Submitted for inter-regional review

COOMET.RI.12.2019 – Georgia

IR CMC REVIEW WORKING GROUP

| Field | Expert | NMI or DI |
| --- | --- | --- |
| Radioactivity | Alekseev Ilya | VNIIM (Russia) |
| Radioactivity | Pilar Oropesa Verdecia | CENTIS (Cuba) |
| Radioactivity | Ivanukovich Alexander | BelGIM (Belarus) |
| Radioactivity | Evseev Vladimir | NSC IM (Ukraine) |
| Dosimetry | Soroka Sergey | BelGIM (Belarus) |
| Dosimetry | Oborin Alexander | VNIIM (Russia) |
| Dosimetry | Gonzalo Walwyn Salas | CHPR (Куба) |
| Neutron Measurements | Moiseev Nikolay | VNIIM (Russia) |
| Neutron Measurements | Didyk Andrey | VNIIM (Russia) |

4.  THE NEXT TC MEETING TIME AND PLACE

Due to the fact that the established tradition of holding TC meetings once per year and a half does not allow for timely response to decisions of the COOMET Committee and Consultative Committees on measurements types, all the next TC meetings should be held annually. The next TC meeting will be held in the autumn of 2019.

TC 1.9 Chairperson

N.Moiseev