|  | Recommendation of COOMET | ### CООМЕТ R/GM/20:2024 |
| --- | --- | --- |
|  | Scales of measurements. Terms and definitions |  |
| ###### Approved in 19th meeting of COOMET Committee (20-21 May 2009, Baku, Azerbaijan). Clarified and supplemented: ###### at the __ meeting of the COOMET Committee (_______________________) |  |  |

1 AREA OF APPLICATION

This recommendation establishes the basic terms and definitions of concepts necessary for the practical application of the theory of measurement scales in legislative and applied metrology.

The established terms are recommended to be used in all types of documentation and scientific and technical literature used in the performance of work in the field of metrology.

The terms defined by the present recommendation are placed in systemized order that

reflects the system of concepts in the scales of measurements theory.

For every concept there is one term defined.

The part of term in round brackets can be omitted when using the term in documents

on standardization. So the part of term out of round brackets represents its short form.

If the square brackets present in terminology statement, it means that the statement

includes two terms with common elements. These terms are included in alphabetical index

separately with the reference to corresponding statement number.

The definitions of terms can be modified by adding specific characteristics, by

revealing the meaning of used terms, by pointing to objects that are bound with defined

concept. But these modifications must not violate the general sense of concepts defined in

present recommendations.

There are equivalents of defined by present recommendations terms in Russian (ru),

German (de) and French (fr) languages.

After the main part of present recommendations there is alphabetical index of terms in

English and alphabetical indexes of terms in Russian, German and French languages also.

The basics of the scales of measurements theory that are important for better

understanding the recommendations are presented in Appendix A.

The present recommendation extend and expand metrological terminology on the

base of scales of measurements theory.

Recommended terms are printed in bold script, their short forms in alphabetical index

are printed in light-face.

The previous version of the recommendation (COOMET R/GM/20:2009) was developed on the basis of RMG 83-2007 [1] and updated by supplementing the "Bibliography" with new documents (publications) [2-10] and correcting bibliographic references in the text with local notes, clarifications and additions.

# 2 BASIC PROVISIONS (TERMS AND DEFINITIONS)

# 2.1 General concepts

2.1.1 Scale (of measurements)

ru  Шкала (измерений)

de  Skala ( der Messungen)

fr   échelle (des Mesurages)

Representation of a set of different realizations of quantitative or qualitative property (or a combination of them) of an object as an accepted by agreement ordered number set or other system of logically related signs (designations).

Notes

1 The concept «scale of measurements» (scale) is not identical to the indication or display component (scale) of a measuring device.

2 There are five main different types of scales: of denominations, of an order, of differences (intervals), of ratios and absolute.

3 The examples of systems of related designations that compose scales of measurements are: set of marks or points to estimate the feature (or ability), set of designations (names) of colours, set of designations (names) of an object state, unity of classification symbols or concepts, set of points in

some model system of coordinates.

4 Scales of differences and ratios are combined inside the term «metric scales».

###### 5 The scales can be one-dimensional and multi-dimensional qualitative, quantitative and combined properties (see Appendix A).

2.1.2 scale of a quantity

ru  шкала величины

de  Skala einer Gröβe

fr  échelle d׳‬ une grandeur‬‬‬

Scale of measurements of quantitative property.

2.1.3 specification for scale (of measurements)

ru спецификация шкалы измерений

de  Spezifikation für skala der Messungen

fr   specification d׳‬ une échelle desmesurages‬‬‬

Accepted by agreement document containing semantic definition of a scale and (or) the description of rules and procedures for the scale realization (or unit if it does exist).

Notes

1 Some metric scales, e.g. scales of mass and length, are specified adequately by standard definitions of units of measurements.

2 Specifications of many (even metrical) scales contain some additional statements along with the definition of units of measurement. For example, the International Temperature Scale ITS-90 contains directions on reproduction of reference points. Specification for scale of light measurements includes not only the definition of luminous intensity (candela), but also tabulated function of spectral luminous efficiency of monochrome radiation on daylight vision.

2.1.4 elements of scale (of measurements)

ru  элемент шкалы измерений

de  Elemente von Skalen der Messungen

fr   еlements d׳‬ une échelle des mesurages‬‬‬

Basic features that describe the scale of measurements: class of equivalence, zero, conventional zero, conventional unit of measurement, natural (dimensionless) unit of measurement, range of the scale, point of the scale.

2.1.5 type of scale (of measurements)

ru  тип шкалы

de  Skalatyp

fr  тype d׳‬ une échelle‬‬‬

Specific set of features on which scales of measurements can be classified. It describes the logical relations between different realizations of a measured property.

2.1.6 reproduction of scale (of measurements)

ru  воспроизведение шкалы измерений

de  Skalareproduction

fr  reproduction de échelle des mesurages

Set of operations in order to reproduce scale of measurements (or a partial range of it) according to its specification.

2.1.7 transmission of scale (of measurements)

ru  передача шкалы измерений

de  Skalatranslation

fr  transmission de échelle des mesurages

Bringing a scale (or a partial range of it) represented by the device under verification (calibration) to conformity with the scale represented by more accurate (reference) standard.

2.2 Types of scales of measurements

2.2.1 scale of denominations

ru  шкала наименований

de  Bezeichnungenscala

fr   échelle d׳‬ un dénominations‬‬‬

Scale of measurements of qualitative property which is fully described by relations of equivalence or difference in realizations of this property.

Notes

1 Sometimes the set of realizations of qualitative property can be ordered by closeness (similarity) of qualitative differences and (or) by quantitative differences in some subsets of property realizations. For example, scales of colour measurements are based on three-coordinate model of colour space in which colours are ordered by differences (qualitative feature) and luminosity (quantitative feature).

2 Scales of denominations have specific set of features: concepts of zero, unit of measurements and dimension have no sense and cannot be used; only isomorphic and homomorphic transformations are permitted; the specification for particular scale cannot be modified. In the most often case

the scale of denominations is defined by a row of “classes of equivalence”.

2.2.2 scale of an order

ru  шкала порядка

de  Ordnungskala

fr   échelle d׳‬ une order‬‬‬

Scale of measurements of quantitative property (quantity), which is fully described by relations of equivalence and ascending (descending) order of different realizations of this property.

Note

Scales of an order have specific set of features: concepts of unit of measurements and dimension do not exist and cannot be used; zero can exist but not necessarily; all monotone transformations are permitted; the specification for particular scale cannot be modified.

2.2.3 scale of differences [intervals]

ru  шкала разностей [интервалов]

de  Skala der Differenzen [Zwischenräume]

fr   échelle d׳‬ un différences [intervalles]‬‬‬

Scale of measurements of quantitative property (quantity), which is fully described by

relations of equivalence and order and by possibility to sumintervals of different realizations of this property.

Note

Scales of differences have specific features: zero and unit of measurements are defined by agreement; the concept of dimension can be applied; linear transformations are permitted; the specification for particular scale can be modified.

2.2.4 scale of ratios

ru  шкала отношений

de  Skala der Verhältnisse

fr   échelle d׳‬ un relations‬‬‬

Scale of measurements of quantitative property (quantity), which is fully described by relations of equivalence, order and proportionality of different realizations of this property. In some cases different realizations of the property can be summed.

Notes

1 Scales of ratios have specific set of features: natural zero exists; unit of measurements is defined by agreement; the concept of dimension can be applied; scaling transformations are permitted; the specification for particular scale can be modified.

2 Scales of ratios are called “proportional scales of ratios” (of the 1st type) if the summation operation cannot be defined because it has no sense. Scales of ratios are called “additive scales of ratios” (of the 2nd type) if the summation operation has sense and can be defined. For example, the scale of thermodynamic temperature is proportional, the scale of mass is additive.

2.2.5 absolute scale

ru  абсолютная шкала

de  Absolute Skala

fr   échelle absolue

Scale of ratios (proportional or additive) of dimensionless quantity.

Notes

1 Absolute scales have specific set of features: natural zero and arithmetical unit of measurements exist and do not depend on chosen system of units; only identical transformations are permitted; the specification for particular scale can be modified.

2 The results of measurements in absolute scales can be presented not only in arithmetical units but also in percents, promille, bits, bytes, decibels (see logarithmic scales).

3 Units of absolute scales can be used in combination with dimensioned units of quantities, e.g. information transmission rate in bits per second.

4 Discrete (countable) scales are a particular sort of absolute scales, the result of measurement in them is expressed by number of particles, quanta or other objects that are equivalent in respect of realization of measured property. For example, scales for electrical charge of atomic nuclei, for number of quanta (in photochemistry), for amount of information. Sometimes some definite number of particles (quanta) is conventionally defined as a unit of measurement (with special denomination) in these scales, e.g. one mol – number of particles equal to Avogadro constant.

2.2.6 absolute limited scale

ru  Абсолютная ограниченная шкала

de  Absolute beschränkte Skala

fr   échelle absolue limité

Absolute scale with range of quantities from zero to one (or to some other limit according to

the specification).

2.2.7 logarithmic scale

ru  логарифмическая шкала

de  Logarithmen Skala

fr   échelle logarithmique

Scale of measurements which is built by logarithmic transformation of measurable quantity.

Note

Usually systems of common or natural logarithms are used to build logarithmic scale; system of binary logarithms can be used also.

2.2.8 logarithmic scale of differences

ru  логарифмическая шкала разностей

de  Logarithmen Skala der Differenzen

fr   échelle logarithmique du différences

Logarithmic scale which is the result of logarithmic transformation of a quantity described by scale of ratios or by intervals in scale of differences, i.e. the scale is defined by dependence L = log

(X/X0), where Х is current and Х0 is adopted by agreement reference value of transformable quantity.

Note

The choice of reference value Х0 defines zero point of logarithmic scale of differences.

2.2.9 logarithmic absolute scale

ru  логарифмическая абсолютная шкала

de  Logarithmen absolute Skala

fr  échelle logarithmique absolue

Logarithmic scale which is the result of logarithmic transformation L= log X of a

dimensionless quantity Х described by absolute scale.

Note

This sort of scales is also called logarithmic scale with floating zero.

2.2.10 biophysical scale

ru  биофизическая шкала

de  Biophysikalische Skala

fr   échelle biophysique

Scale of measurements of a physical factor (stimulus) property which was modified so that the

result of measurements of this property can be used to predict the level or character of reaction of biological object induced by this factor.

2.2.11 one-dimensional scale

ru  одномерная шкала

de  Eindimensionale Skala

fr   échelle monodimensionale

Scale of measurements described by one parameter, in which the results of measurements are represented by one number or one sign (designation).

2.2.12 multidimensional scale

ru  многомерная шкала

de  Veildimensionale Skala

fr   échelle multidimensional

Scale of measurements described by two or more parameters, in which the results of

measurements are represented by blocks of two or more numbers or signs (designations).

Notes

1 Some properties can not be described by one parameter in principle. For example, electrical impedance and complex reflection coefficient are described by two parameters that represent two-dimensional scales. The colour is described by three coordinates in models of colours space that

represent three-dimensional scales.

2 Multidimensional scales can be produced by combination of scales of different types.

3 Often in multidimensional scales some spatial or abstract system of special coordinates is established, e.g. for measurements of velocities and accelerations vectors, for geodesic coordinates the location and orientation angles of the directions (objects) in space.

2.3 Elements of scales

2.3.1 zero of a scale

ru  нуль шкалы

de  Skalanull

fr   zéro d׳‬ une échelle‬‬‬

Initial point in scales of order (some), of intervals, of ratios and absolute.

Note

Zero can be either natural or conventional (adopted by agreement).

2.3.2 natural zero of a scale

ru  естественный нуль шкалы

de  Natürliche Skalanull

fr   naturel zéro d׳‬ une échelle‬‬‬

Zero of a scale that represents infinitesimal (null) quantitative realization of measured

property.

2.3.3 conventional zero of a scale

ru  условный нуль шкалы

de  Verabredete Skalanull

fr   conventionnel zéro d׳‬ une échelle‬‬‬

Zero of scale of differences (intervals) or scale of an order, that represents by agreement null value of measured property (quantity).

Note

The scale can be defined on both sides from conventional zero. For example, in contemporary calendar scale the event of the Birth of Jesus Christ is taken as a conventional zero. So to date some event we usually say “in ... A.D. (anno Domini)” or “in ... B.C. (before Christ)”.

2.3.4 point of a scale

ru  точка шкалы

de  Skalapunkt

fr   point d׳‬ une échelle‬‬‬

One separate number or sign (designation) from specification for scale of measurements.

2.3.5 class of equivalence

ru  класс эквивалентности

de  Äquivalenzklasse

fr   classe d’ équivalence

Subset of realizations of measured property that are conventionally adopted as indistinguishable

in the scale of measurements of this property.

2.3.6 range of a scale of measurements

ru  диапазон шкалы измерений

de  Bereich von Skala der messungen

fr   étendue d׳‬ une échelle des mesurages‬‬‬

Limits of variation in measured property that are covered by the specific given scale representation.

2.3.7 unit of measurement [of quantity]

ru  единица измерений [величины]

de  Einheit [von Größe]

fr   unité de mesure [grandeur]

Fixed quantity magnitude that was conventionally defined as correspondent to numeric value of one in the specific scale of measurements.

Notes

1 The term “unit of quantity” is equivalent to the term “unit of measurement”.

###### 2 The term “unit of a physical quantity” describes narrower concept. It is not recommended for use because it is impossible to define the limits of its application. In [2, 3], the adjective "physical" is not used in connection with the concept of "quantity".

3 The term “unit of measurement” has no sense for properties that are described by scales of denominations and scales of an order.

4 Magnitude of units of quantity described by absolute scales are unambiguously defined by dimensionless character of measured quantities.

5 Multiple prefixes of binary units (for example, bits) have special designations (see [4, 6]).

2.3.8 logarithmic unit of measurement

ru  логарифмическая единица измерений

de  Logarithmen Einheit

fr   unité logarithmique de mesure

Unit of measurement of logarithmic scale.

Note

The following logarithmic units of measurement are commonly used: bel, decibel, log, decilog, neper etc.

2.4 Measurement of properties

2.4.1 object of measurement

ru  объект измерений

de  Objekt der messung

fr   оbject de mesure

Object of activity (body, substance, material, phenomenon, process) one or several specific realizations of quantitative or qualitative properties of which are to be measured.

Note

Not only physical but also non-physical objects can be objects of measurement.

2.4.2 measurable property

ru  измеряемое свойство

de  Meßbare Eigenschaft

fr   propriete de mesure

Common property of objects of measurement which is selected for investigation by measurement.

Note

Not only physical but also non-physical (biological, psychological, social, economical etc.) objects can have quantitative and qualitative measurable properties.

2.4.3 measurable quantity

ru  измеряемая величина

de  Meßbare Größe

fr   grandeur de mesure

Measurable property that is characterized by quantitative differences.

Note

The term “quantity” cannot be applied to qualitative properties which are described by scales of denominations. So the term “property” is wider than the term “quantity”.

2.4.4 measurement

ru  измерение

de  Messung

fr   mesurage

Comparison of concrete realization of measurable property (measurable quantity) with the scale (or its part) of measurements for this property (quantity) in order to obtain the result of measurement (evaluation of a property or value of a quantity).

2.4.5 uniformity of measurements

ru  единство измерений

de  Einheitlichkeit der Messungen

fr   uniformite des mesurages

State of measurements when the results of measurements are expressed in legal units of

measurements (of quantities) or in legal scales of measurements and the uncertainties or margins of errors of results are estimated.

Note

This definition of the term “uniformity of measurements” extends it and covers all types of scales including scales of denominations and scales of an order (see 2.1.1).

2.4.6 value of a quantity

ru  значение величины

de  Größenwert

fr   valeur d׳‬ une grandeur‬‬‬

Expression of quantity magnitude according to appropriate scale in the form of some number of

accepted units, numbers, marks or other signs (designations).

Note

”Evaluation of a property” may be   the analog of the term “value of a quantity” for qualitative properties.

2.4.7 reference quantity value

ru опорное значение величины

de Mengenbezugswert

fr  valeur de référence

The value of a quantity that is used as a basis for comparison with the values of quantities of the same kind.

Notes

1 The reference quantity value may be the true value of the quantity to be measured, in which case it is unknown, or the accepted value of the quantity, in which case it is known [2, 3].

2 The reference value of a quantity with its uncertainty is usually given for:

- material, for example, a certified standard sample;

- devices, for example, a stabilized laser;

- reference measurement procedure;

- the results of the comparison of standards.

3 For qualitative measurable properties, you can use a similar concept — the reference value of a property.

2.4.7.1 true value of a quantity

ru  истинное значение величины

de  Wahrer Größenwert

fr   valeur vraie d׳‬ une grandeur‬‬‬

The value of a quantity that corresponds to the definition of the measured value, i.e. ideally reflects the position on the corresponding scale of the realization of the quantitative property of a particular object of activity.

Notes

1 For qualitative properties, a similar term is "true property evaluation".

2 The definition of the measured value includes the adoption of a certain model of the measuring object, in which the true value is represented by a certain parameter. There is always a threshold discrepancy between the model and the measurement object, which is the cause of the definitional uncertainty of the measured value [ 11].

3 When the definitional uncertainty associated with the measured value is considered negligible compared to the other components of the measurement uncertainty, the measured value can be considered as having "essentially the only" true value. This approach is adopted in GUM [11] and in related documents, where the word "true" is considered superfluous.

2.4.7.2 conventional value of a quantity

ru   принятое значение (величины)

de   Konventioneller Größenwert

fr    valeur conventionnelle d׳ une grandeur

The value of a quantity that is conventionally assigned to a quantity for a given purpose.

Notes

1 Sometimes the accepted value of a quantity is an estimate of the true value of a quantity.

2 The measurement uncertainty associated with the accepted value is often small enough and can be assumed to be zero for a specific purpose. In this case, the concept of the conventional true value of a quantity is used.

2.4.8 conventional true value of a quantity

ru  действительное значение величины

de  Konventioneller wahrer Größenwert

fr   valeur conventionellement vraie d׳‬ une grandeur‬‬‬

Value of a quantity which is so close to true value that it can be used instead of true value for the specific task.

2.4.9 evaluation of a property

ru  оценка свойства

de  Abschätzung einer Eigenschaft

fr   evaluation d׳‬ une propriete‬‬‬

Expression of the place of realization of qualitative property of specific object of measurement in appropriate scale of denominations.

Note

In some cases it is acceptable and suitable to use the term “value” instead of

“evaluation” for qualitative properties.

2.4.10 true evaluation of a property

ru  истинная оценка свойства

de  Wahr Abschätzung einer Eigenschaft

fr   valeur evaluation d׳‬ une propriete‬‬‬

Evaluation of a property that could ideally reflect the realization of qualitative property of specific object of measurement in appropriate scale.

2.4.11 conventional evaluation of a property

ru  действительная оценка свойства

de  Konventionell Abschätzung einer Eigenschaft

fr   evaluation conventionellement d׳‬ une propriete‬‬‬

Evaluation of a property which is so close to true evaluation that it can be used instead of true evaluation for the specific task.

2.4.12 method of measurement

ru  метод измерения

de  Meßmethode

fr   methode de mesure

Technique or totality of techniques which are used to establish the fact of equivalence between concrete realization of measurable property (value) and the element of scale of measurements for this property.

2.4.13 result of measurement

ru  результат измерения

de  Meßergebnis

fr   resultat d ́un mesurage

Value of a quantity or evaluation of a property that was obtained by the way of measurement.

Notes

1 Arithmetic mean value of several uniformly precise acts of measurement is taken most often as a result of measurement in scales of differences (intervals), of ratios and absolute scales.

2 In scales of order, the median of the results of a number of repeated measurements can be taken as the measurement result, but the arithmetic mean cannot be taken.

3 A result of measurement in scales of denominations reflects the equivalence of the specific realization of measured property to the point or class of equivalence of appropriate scale.

4 The result of measurement is invalid without information about its uncertainty or error margins.

Therefore, in general, in accordance with VIM 3 [2], the manual [11] and the recommendations [3], the measurement result is a set values of quantity or property estimates on a scale corresponding to the uncertainty of this measurement, along with any other available and significant information.

2.5 Measuring instruments and etalons

2.5.1 measuring instrument

ru  средство измерений

de  Meßmittel

fr   instrument de mesure

Object having normalized metrological characteristics and designed for execution of measurements. Measuring instrument reproduces and/or keeps some part (or point) of the scale.

2.5.2 material measure

ru  мера

de  Maßverkörperung

fr   mesure materialisee

Measuring instrument that keeps and reproduces one or more points of the scale of measurements.

Note

This term can be used not only in scales of quantitative properties (material measure of a value) but also in scales of qualitative properties, e.g. material measure of the colour.

2.5.3 measuring device

ru  измерительный прибор

de  Meßgerät

fr   appareil de mesure

Measuring instrument (or a standard) that is designed to obtain the value of measured quantity or the evaluation of measured property in certain range (part) of measurements scale.

2.5.4 measuring transducer

ru  измерительный преобразователь

de  Meßumformer

fr   transducteur de mesure

Measuring instrument (or its part) which function is to get information about measured

quantitative or qualitative property and to transform it into the suitable form for future processing, storage, consequent transformation, indication or transmission.

2.5.5 comparator

ru  компаратор

de  Komparator

fr   сomparator

Object that is designed to compare realizations of measurable property (quantity).

Note

There exist comparators not only of quantitative properties (values) but of qualitative properties as well, e.g. comparators of the colour.

2.5.6 standard [etalon]

ru  эталон

de  Normal

fr   etalon

Object that is designed and legalized to reproduce and/or store the scale, the part of it or the magnitude of the unit of measurements and to perform a transmission of them to measuring instruments.

Note

The standard, as a basis for comparison, implements a specification of the measurement scale (see 2.1.3) or a semantic definition of a quantity with established values of properties (quantities) and their accuracy indicators (uncertainties, estimates of the limits of reproduction errors).

2.5.7 etalon of scale

ru  эталон шкалы измерений

de  Normal einer Skala

fr   etalon d ́un échelle

Etalon that reproduces all scale in the whole or some part of it.

Notes

1 An etalon (measurement standard) can reproduce one point of the scale (one fixed value of a quantity or a qualitative property) – see 2.5.8.

2 Etalons of scales of denominations and scales of an order usually must reproduce in the whole all used parts of the scale.

2.5.8 etalon of a quantity

ru эталон величины

de Normal einer Größe

fr etalon d´un grandeur

Etalon that reproduces one or

several values of measured quantity (points of the scale).

N o t e

1 An etalon (standard) can reproduce one point of the scale (one fixed value of a quantity or a qualitative property) – see 2.5.8.

2 Etalons of scales of denominations and scales of an order usually must reproduce in the whole all used parts of the scale.

2.6 Errors and uncertainties

2.6.1 error (of measurement)

ru погрешность (результата) измерени

de Meßabweichung

fr erreur (de mesure)

Deviation of result of measurement from true value of a quantity or from true

evaluation of a qualitative property.

N o t e s

1 In practice estimation of an error is performed by means of replacement

of true value of a quantity or true evaluation of a property by conventional

true value of a quantity or conventional true evaluation of a property

respectively the reference value of a quantity or property (see 2.4.7).

2 Error in two-dimensional and multi-dimensional scales is described by

the deviation of the point of the scale that corresponds to the result of

measurement from the point of the scale that corresponds to true value (true

evaluation) in the model space from the point of the scale corresponding to the reference value in a particular model space.

3 In scales of ratios and absolute scales the term “absolute error” is used to

distinguish it from “relative error”.

2.6.2 relative error

ru относительная погрешность (измерения)

de Relative Meßabweichung

fr erreur relative

Ratio of an error of measurement to reference value of a quantity.

N o t e s

1 The concept of relative error can be applied to measurements in scales of

ratios and absolute scales. It can be applied also to intervals of values in

scales of differences (intervals). But it cannot be applied to the values in

scales of differences themselves. For example, it is impossible (it has no

sense) to show in percents an error of measurement of Celsius temperature

or an error of dating of event.

2 The concept of relative error cannot be used in scales of an order and

scales of denominations.

2.6.3 errors of scale reproduction

ru погрешность воспроизведения шкалы

de Meßabweichungen einer Skalareproduction

fr erreurs d ׳ une reproduction de échelle

The deviation (difference) of the reproduced value of the property (quantity) scale point from its reference value.

N o t e s

Primary standards and primary reference procedures (methods) are recommended to be characterized not by errors, but by estimated uncertainties in reproducing scale points.

2.6.4 errors of scale transmission

Ru погрешность передачи шкалы

de Meßabweichungen einer skalatranslation

fr erreurs d ׳une transmission de echelle

The error in the procedure for transmitting the values of the scale points of the property (quantity), including errors in the transmission method and the standard from which the transmission is carried out, as well as random errors of the standard (measuring instrument) to which the scale is transmitted.

2.6.5 uncertainty of measurement

Ru неопределенность (результата) измерений

de Meßunsicherheit

fr incertitude de mesure

Region (section, area) of a scale of measurements, in which evaluation of a property or

value of a quantity is supposed to be contained according to possible dispersion of measurement results.

N o t e s

1 In one-dimensional scales of ratios, of intervals and absolute scales the

uncertainty is usually characterized by the parameter derived from root

mean square deviation: standard uncertainty and expanded uncertainty as

described in “Guide to the expression of uncertainty in measurement” [11].

2 In two-dimensional and multi-dimensional scales the region (area) of a

scale that characterizes the uncertainty of measurement presents multidimensional

(two-dimensional) region in corresponding model space

around the point of the scale which represents the result of measurement [8-10].

3 In scales of an order and of denominations the uncertainty of

measurement can be described by the spread of distribution but not by

standard (or expanded) uncertainty [5, 10].

2.6.6 uncertainty of scale reproduction

ru неопределенность воспроизведения шкалы

de Unsicherheit einer Skalareproduction

fr incertitude d ׳ une reproduction de échelle

The uncertainty of the values of a property (quantity) when reproducing scale points is the procedure for implementing the specification of the measurement scale (see 2.1.3) or the semantic definition of a value.

2.6.7 uncertainty of scale transmission

ru неопределенность передачи шкалы

de Unsicherheit einer Skalatranslation

fr incertitude d ׳ une transmission de échelle

The uncertainty of reproduction of the transmitted scale points in combination with the uncertainties of the transmission method, as well as the uncertainty caused by the random error of the standard (measuring instrument) to which the scale is transmitted.

Alphabetic index of equivalents of the terms in Russian

В

величина измеряемая  2.4.3

воспроизведение шкалы измерений  2.1.6

Д

диапазон шкалы измерений  2.3.6

Е

единица величины  2.3.7

единица измерений  2.3.7

единица измерений логарифмическая  2.3.8

единство измерений  2.4.5

З

значение величины   2.4.6

значение величины действительное  2.4.8

значение величины истинное  2.4.7.1

значение величины опорное   2.4.7

значение величины принятое   2.4.7.2

И

измерение  2.4.4

интервал  2.2.3

К

компаратор  2.5.5

класс эквивалентности  2.3.5

М

мера  2.5.2

метод измерений  2.4.12

Н

неопределённость воспроизведения шкалы  2.6.6

неопределённость измерений 2.6.5

неопределённость передачи шкалы 2.6.7

неопределённость результата измерений   2.6.5

нуль шкалы   2.3.1

нуль шкалы естественный  2.3.2

нуль шкалы условный  2.3.3

О

объект измерений  2.4.1

оценка свойства  2.4.9

оценка свойства действительная  2.4.11

оценка свойства истинная  2.4.10

П

передача шкалы измерений  2.1.7

погрешность воспроизведения шкалы  2.6.3

погрешность передачи шкалы  2.6.4

погрешность измерения  2.6.1

погрешность измерения относительная  2.6.2

погрешность относительная  2.6.2

погрешность результата измерения  2.6.1.

преобразователь измерительный  2.5.4

прибор измерительный  2.5.3

Р

результат измерения  2.4.13

С

свойство измеряемое 2.4.2

спецификация шкалы измерений  2.1.3

средство измерений  2.5.1

Т

тип шкалы 2.1.5

точка шкалы 2.3.4

Ш

шкала  2.1.1

шкала абсолютная  2.2.5

шкала биофизическая  2.2.10

шкала величины   2.1.2

шкала измерений  2.1.1

шкала интервалов  2.2.3

шкала логарифмическая  2.2.7

шкала абсолютная логарифмическая  2.2.9

шкала абсолютная ограниченная  2.2.6

шкала разностей логарифмическая  2.2.8

шкала многомерная  2.2.12

шкала наименований  2.2.1

шкала одномерная  2.2.11

шкала отношений  2.2.4

шкала порядка  2.2.2

шкала разностей  2.2.3

Э

элемент шкалы измерений  2.1.4

эталон величины  2.5.8

эталон  2.5.6

эталон шкалы измерений  2.5.7

Alphabetic index of equivalents of the terms in German

A

Abschätzung eine Eigenschaft  2.4.9

Absolute beschränkte Skala  2.2.6

Absolute Skala  2.2.5

B

Bereich von Skala der messungen  2.3.6

Beschränktlskala absolute  2.2.6

Bezeichnungenskala  2.2.1

Biophysikalische Skala  2.2.10

E

Eindimensionale  Skala  2.2.11

Einheit (von Größe)  2.3.7

Einheitlichkeit der Messungen  2.4.5

Elemente von Skalen der Messungen  2.1.4

G

Größenwert  2.4.6

K

Klasse Äquivalent  2.3.5

Komparator  2.5.5

Konventionell Abschätzung einter Eigenschaft  2.4.11

Konventioneller Größenwert  2.4.7.2

Konventioneller wahrer Größenwert  2.4.8

L

Logarithme Skala der Differenzen  2.2.8

Logarithmen Einheit  2.3.8

Logarithmen Skala  2.2.7

Logarithmen absolute Skala  2.2.9

M

Maßverkörperung  2.5.2

Meßabweichung  2.6.1

Meßabweichungen einer skalareproduction  2.6.3

Meßabweichungen einer skalatranslation  2.6.4

Meßbare Eigenschaft  2.4.2

Meßbare Größe  2.4.3

Meßergebnis  2.4.13

Meßgerät   2.5.1, 2.5.3

Meßmethode  2.4.12

Meßumformer  2.5.4

Mengenbezugswert 2.4.7

Messung  2.4.4

Meßunsicherheit  2.6.5

N

Natürliche Skalanull  2.3.2

Normal  2.5.6

Normal einer Skala   2.5.7

Normal eines Größe   2.5.8

O

Objekt der messung  2.4.1

Ordnungskala  2.2.2

R

Relative Meßabweichung  2.6.2

S

Skala (der Messungen)  2.1

Skala der Differenzen (Zwischenräume)  2.2.3

Skala der Verhältnise  2.2.4

Skala einer Gröβe  2.1.2

Skalanull  2.3.1

Skalapunkt  2.3.4

Skalareproduction  2.1.6

Skalatranslation 2.1.7

Skalatyp   2.1.5

Spezifikation fǖr Skala der messungen  2.1.3

U

Unsichertheit einer Skalareproduction  2.6.6

Unsichertheit einer skalatranslation  2.6.7

V

Veildimensionale Skala  2.2.12

Verabredete Skalanull  2.3.3

W

Wahr Abschätzung eine Eigenschaft  2.4.10

Wahrer Größenwert  2.4.7.1

Z

Zwischenräume  2.2.3

Alphabetic index of the terms in English

A

absolute limited scale 2.2.6

absolute scale 2.2.5

B

biophysical scale 2.2.10

C

class of equivalence 2.3.5

comparator  2.5.5

conventional evaluation of a property  2.4.11

conventional true value of a quantity   2.4.8

conventional value of a quantity   2.4.7.2

conventional zero of a scale  2.3.3

E

elements of measurements scales 2.1.4

error (of measurement)  2.6.1

errors of scale reproductiocalen  2.6.3

errors of scale transmission 2.6.4

etalon of a quantity 2.5.8

etalon of a scale  2.5.7

evaluation of a property  2.4.9

L

logarithmic scale  2.2.7

logarithmic absolute scale  2.2.9

logarithmic scale of differencaleces 2.2.8

logarithmic unit of measurecalement  2.3.8

M

material measure  2.5.2

measurable property  2.4.2

measurable quantity  2.4.3

measurement  2.4.4

measurements scale  2.1.1

measuring instrument  2.5.1, 2. 5.3

measuring transducer  2.5.4

method of measurement  2.4.12

multidimensional scale  2.2.12

N

natural zero of a scale  2.3.2

O

object of measurement 2.4.1

one-dimensional scale 2.2.11

P

point of a scale  2.3.4

R

range of measurements scale  2.3.6

reference value of a quantity   2.4.7

relative error  2.6.2

reproduction of measurements scale  2.1.6

result of measurement  2.4.13

S

scale  2.1.1

scale of a quantity  2.1.2

scale of an order  2.2.2

scale of denominations  2.2.1

scale of differences (intervals) 2.2.3

scale of ratios  2.2.4

specification for scale of measurements 2.1.3

standard, etalon   2.5.6,  2.5.7

T

transmission of measurement scale  2.1.7

true evaluation of a property  2.4.10

true value of a quantity  2.4.7.1

type of a scale  2.1.5

U

uncertainly of measurement  2.6.5

uncertainly of scale reproduction  2.6.6

uncertainty of scale transmission  2.6.7

uniformity of measurements  2.4.5

unit of measurement (of quantity)  2.3.7

V

value of a quantity  2.4.6cale

Z

zero of a scale  2.3.1

Alphabetic index of equivalents of the terms in French

C

classe une equivalence                  2.3.5

comparator 2.5.5

conventionnel zero d׳ une echelle 2.3.3

E

echelle (des Mesurages) 2.1.1

echelle absolue 2.2.5

echelle absolue limite 2.2.6

echelle biophysique 2.2.10

echelle d´un etalon 2.5.7

echelle d׳ un denominations 2.2.1

echelle d׳ un differences (intervalles) 2.2.3

echelle d׳ un relations 2.2.4

echelle d׳ une grandeur 2.1.2

echelle d׳ une order 2.2.2

echelle logarithmique 2.2.7

echelle logarithmique absolue 2.2.9

echelle logarithmique du differences 2.2.8

echelle monodimensionale 2.2.11

echelle multidlimensionale 2.2.12

elements d׳ un echelles des mesurages 2.1.4

erreur (de mesure) 2.6.1

erreur relative 2.6.2

erreurs d׳ une reproduction de echelle 2.6.3

erreurs d׳ une transmission de echelle 2.6.4

etalon 2.5.6

etalon d´un grandeur 2.5.8

etendue d׳ une echelle des mesurages 2.3.6

evaluation conventionellement d׳ une propriete 2.4.11

evaluation d׳ une propriete 2.4.9

G

grandeur de mesure 2.4.3

I

incertitude d׳ une reproduction de echelle 2.6.6

incertitude d׳ une transmission de echelle 2.6.7

incertitude de mesure 2.6.5

instrument de mesure 2.5.1, 2.5.3

M

mesurage 2.4.4

mesure materialisee 2.5.2

methode de mesure 2.4.12

N

naturel zero d׳ une echelle 2.3.2

O

object de mesure 2.4.1

P

point d׳ une echelle 2.3.4

propriete de mesure 2.4.2

R

reproduction de echelle des mesurage 2.1.6

resultat d´un mesurage 2.4.13

S

specification d׳ une echelle des mesurages 2.1.3

T

transducteur de mesure 2.5.4

transmission de echelle des mesurages 2.1.7

type d׳ une echelle 2.1.5

U

uniformite des mesurages 2.4.5

unite de mesure (grandeur) 2.3.7

unite logarithmique de mesure (grandeur) 2.3.8

V

valeur conventionnelle d׳ une grandeur   2.4.7.2

valeur conventionellement vraie d׳une grandeur 2.4.8

valeur d׳ une grandeur 2.4.6

valeur de référence   2.4.7.

valeur evaluation d׳ une propriete 2.4.10

valeur vraie d׳ une grandeur 2.4.7.1

Z

zero d׳ une echelle 2.3.1

Appendix А

(for reference)

Elements of the theory of scales of measurements

The term “scale” in metrology practice has at least two different meanings. Firstly the

scale, or to be more precise the scale of measurements, is an abstract concept that is defined in

present Recommendation. Secondly the scale is readout device of analog measuring

instruments. In this Recommendation the term “scale” is used in the first meaning only.

This appendix describes basic elements of the theory of scales of measurements which

are necessary to understand and to use them, along with some examples of scales which are

widely used in practice.

There are various properties of objects, substances, phenomena, processes that are

measured. Measured properties are named as mass, time interval, thermodynamic

temperature, hardness, colour etc. Some properties are realized quantitatively (length, mass,

temperature etc.) but the others are realized qualitatively.

Often, the properties of measurement objects are logically determined by a set of quantitative and (or) qualitative properties, i.e. they are combined multidimensional properties. For example, color is not a scalar quantity, since an expression like "red is greater (less) than blue" does not make sense, it is determined by three-dimensional models [12].

Measured quantitative properties are called measured values. The variety (quantitative or qualitative) of realizations of any property produces the set. This set can be represented by set of numbers or in general case by system of conventional notations thus representing the scale of measurements for this property. Examples of systems of notations are: set of colours denominations (names), unity of classification symbols or concepts, set of marks (points) for evaluation of object state, set of real numbers etc. Elements of the set of property realizations are bound by certain logical relations. Relations of this kind can be “equivalence” (equality), “difference”, “similarity”

(closeness) between two elements, their quantitative distinguishability (“more”, “less”), real

feasibility of summing, subtraction, multiplication, division operations on elements of the set

etc. These particular features of properties define the type of corresponding scales of

measurements.

According to logical structure of properties realizations there are five basic types of

scales of measurements in the measurements theory distinguished: scales of denominations,

scales of an order, scales of differences (intervals), scales of ratios and absolute scales [1-3, 5, 10]. Scales of each type have certain characteristics, main of them are examined below.

Scales of denominations reflect qualitative properties. Their elements are characterized only by relations of equivalence (equality), difference and similarity of particular qualitative realizations of properties. It is impossible to define unit of measurements concept in scales of denominations and consequently a concept of dimension also, zero element is absent in this type of scales. Nevertheless, some statistic operations are possible in processing the results of measurements. For example, it is possible to find modal or the most numerous class of equivalence by the results of measurements.

Examples of scales of denominations

– Scales of colour measurements – colorimetry systems standardized by Commission International

d'Eclairage (CIE). In the most widely used standard colorimetry system CIE of the year 1931 the colour is

defined by three colour coordinates X, Y, Z in model three-dimensional non-Euclidean space using spectral

characteristics of optical emission sources, reflecting and transparent objects and empiric standardized

functions of colour addition (see publication 15 CIE [12]).

– Geodesic scales for description of location on the Earth in prescribed coordinate systems

(geodesic coordinates, astronomic coordinates, geocentric coordinates, flat rectangular geodesic coordinates

and others). The initial international terrestrial coordinate system is defined by Resolution 9 (24th GCMW, 2011) [4], according to which the International Terrestrial Coordinate System ITRS, defined by the International Union of Geodesy and Geophysics IUGG and implemented by the International Earth Rotation and Coordinate Systems Service IERS, is adopted as the only international coordinate system for terrestrial reference systems (coordinate bases) in all metrological annexes (for the full text of Resolution 9 , see Appendix B).

– Scales of space symmetry (scale of crystal symmetry groups etc.).

– Scales of odours.

– Scale of human blood group taking into consideration Rhesus factor etc.

Scales of an order describe the properties, for which not only the relations of

equivalence but also the relations of an order by ascending or descending of quantitative

property realizations have sense. Very specialized scales of an order are widely used in

methods of testing various products.

In this type of scales it is impossible to introduce units of measurements because they

are principally non-linear, i.e. it is logically impossible to establish equality of intervals in

different parts of the scale. The results of measurements in these scales are expressed in

numbers, marks, points, indexes, levels, but not in units of measurements. Though the results

of measurements are often designated by continuous set of real numbers, it is impossible to

imply proportionality of these values (because it is logically impossible to define how much

times one property realization is more or less than the other). The results of measurements in

marks, points, levels are expressed in discrete natural numbers. Scales of an order allow

monotone transformations and sometimes zero can exist.

# Examples of scales of an order [5, 10]

– Scales of material hardness: for metals (international scales of Brinell, Rockwell, Vickers, Shore),

minerals, rubber, plastics etc.

– Scales of intensity and levels for earthquakes.

– Scales of strength of wind and marine surface state (Beaufort number etc.).

– Scales of whiteness for different objects (materials, stuff, goods), e.g. for the paper, wood, flour etc.

– Scales of numbers for photosensitivity of light-sensitive materials.

– Scales of loudness, levels of loudness.

– Scales of aroma strength and taste of water.

– Scales of octane and cetane numbers for petrol.

– Scale of falling number for cereals.

– Scale of evaluation the events at nuclear power plants.

– Scales of acid, iodine, bromine, permanganate, copper, chlorine, bentonite, formolite, peroxide,

carbonyl, essential and other numbers for different materials or products.

Scales of differences (intervals) differ from scales of an order by the fact that for

properties described by them not only the relations of equivalence and order have sense, but

also equality and proportionality of intervals (differences) between different quantitative

realizations of the property. It means that the intervals can be summed or multiplied by some coefficient but the values cannot be summed or multiplied because these operations have no sense. There is a typical illustration – scale of time intervals. Time intervals (e.g. periods of work, periods of education) can be summed, it is possible to say that someone is twice older than the other, but it has no sense to sum the dates of two events. Another example is the scale of length (distance) or in other words scale of space extension intervals which is defined by counting out from particularly chosen point in space (conventional zero) to the other point. Practical scales of temperature (Celsius, Fahrenheit, Reaumur) with conventional zero are also scales of this type. Scales of differences have conventional units of measurements and conventional zero based on some reference points. Linear transformations are permitted in scales of differences, and it is possible to calculate average of a distribution, standard deviation and other statistics parameters.

Examples of scales of differences [4, 10]

– International scale of uniform atomic time TAI (Temps Atomique International), in which the

magnitude of unit conforms to the definition of the second in International System of Units SI [4].

– Universal Time scale UT0, in which the duration of second is equal to mean solar second [4].

– Universal Time scale UT1 that differs from UT0 by correction for the effect of polar motion. [4].

– Universal Time scale UT2, that differs from UT1 by filtering out periodic seasonal variations of the

Earth’s rotation[4].

– Coordinated Universal Time scale UTC, in which the value of second is the same as in TAI, but the

reference moment can vary in steps of one second in order to keep UTC within 0,9 seconds from UT1 [4].

Note: The current definitions of time scales are given in Resolution 2 of the 26th meeting of the CGPM (its contents are reflected in Appendix 2 SI Brochure 9e édition 2019 [4]). The translation of the resolutions into Russian is published on the COOMET website: http://coomet.org/DB/isapi/cmt_docs/2019/01/26th-CGPM-Resolutions%202018.pdf).

– Calendars (Gregorian, Julian, Moslem, lunar, etc.) [10].

– Celsius temperature scale, in which the unit of measurements (centigrade degree) is equal to Kelvin

and thermodynamic temperature 273,16 K is accepted as conventional zero [4].

– Scale of oxidizing potentials of water solutions.

Scales of ratios. In these scales relations of equivalence, order and proportionality

exist between different quantitative realizations of a property, also operations of subtraction,

multiplication by factor and division can be applied (scales of ratios of the 1st type –

proportional). In many cases different realizations of a property can be summed (scales of

ratios of the 2nd type – additive).

Conventional units and natural zeroes exist in scales of ratios, scaling transformations

are permitted. These scales are widely used in science and technology, it is possible to

calculate average of a distribution, standard deviation, standard and expanded uncertainty and

other statistics parameters.

Examples of scales of ratios

– Scale of a mass (additive).

– Scale of frequencies, in which the magnitude of unit conforms to the definition of Hertz in SI.

– Scale of thermodynamic temperature (proportional), in which the magnitude of unit conforms to

the definition of Kelvin in SI (the International Temperature Scale ITS-90 is maximally close to it and defines

a set of reference points).

– Scale of luminous intensity, in which the value of unit conforms to the definition of candela in SI

with application of standardized by Commission International d'Eclairage (CIE) [13] empiric function of

spectral luminous efficiency of monochrome radiation on daylight vision for different by spectrum optical

radiations. This scale is a source for scales of all photometric quantities.

– Scales of doses (absorbed, equivalent) and dose rates of ionizing radiations.

– Practical Salinity Scale (PSS-78) [14].

– International Sugar Scale [15].

– Scales of water hardness.

Absolute scales have all features of scales of ratios, but additionally there exists

natural unambiguous definition for unit of measurement in them. These scales are used for

measurements of relative quantities (ratios of similar quantities): amplification and

attenuation coefficients, efficiency factor, reflection and absorption coefficients, amplitude

modulation factor, etc.

Examples of absolute scales  [5, 10]

– Scales of plane angles with units of measurements according to SI – radian and angular degree.

– Scale of solid angles with unit of measurements according to SI - steradian.

– Scales of coefficients and factors: of amplitude modulation, of nonlinear distortion, of

amplification, of attenuation, of reflection.

– Scale for Q-factor of oscillating systems.

– Scale of relative dielectric constant.

– Scales of brilliance.

– Scales of frequency intervals used in acoustic measurements.

– Scales of humidity.

Most of properties are described by one-dimensional scales, but there exist properties

described by multidimensional and combined scales: three-dimensional scales of coloures in colorimetry, two-dimensional scales of electric impedances, navigation and geodetic scales of directions (angles) of orientation in space, multidimensional scale of Earth rotation parameters, in which rotation axis position in the Earth’s body, the direction of rotation axis in cosmic space, the value and variations of angular velocity of the Earth’s rotation are defined. Main characteristics and specific features of scales main types are systemized in Table A.1

T a b l e A.1 – Characteristics and specific features of scales main types

| Feature | Type of measurement scale |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Of denomi- nations | Of an order | Of differences (intervals) | Of ratios |  |  |  | Absolute |
|  |  |  |  | 1st type |  | 2nd type |  |  |
| Allowed logical and mathematical relations between realizations of properties | Equivalence, distinction | Equivalence, distinction, order | Equivalence, order, intervals can be summed | Equivalence, order, proportionality |  | Equivalence, order, quantities can be summed |  | Equivalence, order, proportionality, sometimes quantities can be summed |
| Zero | Has no sense | Not necessary | Conventional | Defined naturally |  |  |  |  |
| Unit of measurements | Has no sense |  | Magnitude is defined conventionally |  |  |  | Magnitude is defined by natural criterion |  |
| Allowed transformations | Isomorphic mapping | Monotone transformations | Linear transformations |  | Multiplication by coefficient |  | Absent |  |

Practical realization of measurement scales is achieved by the way of standardization

of scales and units of measurement as well as methods and conditions (specifications) for

their unambiguous reproduction if needed. Scales of denominations and of an order can be

realized also without any specialized technical standard devices (Linnaeus classification scale,

scale of odours, Beaufort scale) but if it is necessary to build etalons, than they reproduce all

practically used part (region) of the scale (e.g. hardness etalon). Any changes in specification

that defines scale of denominations or of an order means in practice introduction of a new

scale. Scales of differences and of ratios (metric scales) corresponding to the SI are

reproduced by etalons as a rule. Etalons for these scales of measurements can reproduce only

one point of the scale (mass etalon), some region of the scale (length etalon) or practically full

scale (time etalon).

Normative documents in metrology usually consider establishment and reproduction

of measurement units only. In practice even for quantities that correspond to basic units in the

SI (second, Kelvin, candela etc.) etalons keep and reproduce scales in addition to units (scales

of atomic and astronomical time, temperature scale ITS-90 etc.) [4]. In any variant of etalon

construction the tracebility chain makes provision for reproduction of all practically needed

parts (regions) of scales. Absolute scales can rely on etalons which reproduce any region of

them (like etalons of metric scales), but also can be reproduced without etalons (efficiency

factor, amplification coefficient). Specific features of scales reproduction (realization) are

systemized in Table А.2.

T a b l e А.2 – Specific features of scales realization

| Feature | Type of measurement scale |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Of denomi- nations | Of an order | Of differences (intervals) | Of ratios | Absolute |
| Establishment of measurements units | Impossible to establish units of measurement in principle. |  | Possible to establish units of measurement |  |  |
| Necessity of the standard for scale reproduction | Scales can be reproduced without special technical standard device |  | Most of scales can be reproduced only by means of special technical standard device |  | Scales can be reproduced without standards |
| Standard (if exists) must reproduce | All practically used regions of the scale |  | Some part or point of the scale and conventional zero | Some part or point of the scale | Obligatory requirements are absent |

Scales of ratios, absolute scales and intervals in scales of differences can be subject to

logarithmic transformation that leads to change in the type of a scale. These scales are called

logarithmic. Logarithmic scales based on application of common and natural logarithms

systems and binary logarithms as well are widely used.

Operation of taking a logarithm can be applied to dimensionless quantities only, so

before taking a logarithm dimensional quantity must be transformed into dimensionless by its

division by conventionally adopted fixed (reference) value of the same quantity, than a

logarithm of obtained dimensionless quantity can be calculated.

Depending on the type of scale that was a subject to logarithmic transformation

logarithmic scales can be one of two types. Logarithmic transformation of an absolute scale

results in absolute logarithmic scale also called logarithmic scale with floating zero, because

reference value is not fixed in them. Examples of these scales are scales of signal

amplification (attenuation) in decibels. Operations of summing and subtraction are permitted

on values of quantities in absolute logarithmic scales.

Logarithmic transformation of scales of ratios and of intervals in scales of differences results

in logarithmic scales of intervals with fixed zero corresponding to reference value of initial

quantity. Most frequently used reference values are 1 mW, 1 V, 1 μV in radiotechnics, 20 μPa

in acoustics etc. In general case no one arithmetic operation can be applied to values in these

scales. Quantities expressed in values in these scales can be added or subtracted by means of

obtaining their antilogarithms, performing necessary operation and taking the logarithm of the

result.

Examples of logarithmic scales [5, 10]

– Scales of sound pressure level А, В, С and D standardized internationally. Sound pressure level in

these scales is conventionally expressed in logarithmic scales (in decibels relative to reference value 2·10-5

Pa).

– Scales of measurements for irritant action of noise (for noisiness and for perceived noise levels)

standardized internationally.

– Audiometric scales (for measurement of hearing acuity and grades of hearing loss).

– Psophometric scales (for measurement of noise interference in communication lines).

– Scale of hydrogen ion exponent рН of water solutions which is reproduced with the use of set of

reference solutions.

– Scales of ionometric exponents.

In practical metrology there is a variety of scales which describe the reactions

(responses) of biological objects (a human, first of all) on actuating physical factor. They

include luminous measurements and colorimetric scales, scales of sounds perception, scales

of equivalent doses of ionizing radiation etc. These scales are commonly named as

biophysical scales.

Biophysical scale is a scale of measurements for properties of physical factor

(stimulus) modified in such a way that the result of measurement of this property can be used

to predict the level or the character of biological object’s response on the influence of that

factor. These scales are resulting from application of a model that modifies (transforms) the

measurement results of stimulus property so that univocal correspondence can be reached

between the results of measurement and the characteristics of biological reaction

(homomorphic image of the set of stimuli on the set of responses). At the same time

equivalent responses can correspond to some subset of stimuli. This modified scale of stimuli

by its logical structure approaches to the structure of responses scale thus acquiring some

prognostic meaning. Nevertheless, as a rule, biophysical scale of stimuli and a scale of

correspondent responses are of different types, so logical relations in the scale of stimuli

cannot be simply extended onto prognostic judgments about responses. For example, the scale

of brightness from the point of view of stimuli is unlimited additive scale of ratios, but from

the point of view of human being response it is a scale of an order in a bounded below and

above diapason of stimulus value [10, 13].

Appendix B

(for reference)

Resolution 9, 24th meeting CGPM, 2011 г.

(https://www.bipm.org/documents/20126/30876806/CGPM24.pdf/7c41c2c1-71db-5ef9-4cdd-387279251bac)

24th meeting of the CGPM - Resolutions · 547

• On the adoption of a common terrestrial reference system

Resolution 9

The General Conference on Weights and Measures (CGPM), at its 24th meeting,

considering

• that a significant number of global navigation satellite systems (GNSS) now exist and that in the future there may be more,

• the proliferation of time and geodesy reference systems in use in these navigation systems, which creates ambiguities for users with regard to the interpretation of navigation and timing solutions, and which renders interoperability between the systems more difficult,

• the existence of the International Terrestrial Reference System (ITRS),

• that the adoption of a common reference system would lead to benefits for users regarding unification of navigation and timing solutions and systems interoperability,

recommends that the ITRS, as defined by the International Union of Geodesy and Geophysics (IUGG) and realized by the International Earth Rotation and Reference Systems Service (IERS), be adopted as the unique international reference system for terrestrial reference frames for all metrological applications.

https://www.bipm.org/documents/20126/30876806/CGPM24.pdf/7c41c2c1-71db-5ef9-4cdd-387279251bac

CONTENTS

1 Area of application...........................................

# 2 Basic provisions (terms and definitions) ......................

2.1 General concepts.............................................

2.2 Types of scales of measurements.........................................

2.3 Elements of scales of measurements....................................

2.4 Measurement of properties...........................................

2.5 Means of measurements and standards................................

2.6 Error and uncertainty   ....................

Appendix A (for reference).  Elements of the theory of scales of measurements........

Alphabetic index of the terms in English..................

Alphabetic index of equivalents of the terms in Russian ……..

Alphabetic index of equivalents of the terms in German ……...

Alphabetic index of the terms in English……………...

Alphabetic index of equivalents of the terms in French ……….

Appendix B (for reference). Resolution 9, 24th meeting CGPM, 2011 г. ….

Contents ………………………………………………….

Bibliography.................................................

Information data ………………

Bibliography

| [1] РМГ 83-2007. | ГСИ. Шкалы измерений. Термины и определения (взамен МИ 2365-96 ГСИ. Шкалы измерений. Основные положения. Термины и определени7я. – М.: ВНИИФТРИ. 1996. – 34 с.), GSI. Measurement scales. Terms and definitions (instead of MI 2365-96 GSI. Measurement scales. The main provisions. Terms and definitions. – M.: VNIIFTRI. 1996. – 34 p.), |
| --- | --- |
| [2] VIM 3 | International vocabulary of metrology: Basic and general concepts and associated terms. JCGM 200:2008 (E/F). |
| [3] РМГ 29-2013 | ГСИ. Метрология. Основные термины и определения. GSI. Metrology. Basic terms and definitions. |
| [4] Le Système international d’unités (SI), English version | The International System of Units (SI), 9e édition 2019, Édité par le BIPM, Pa-villon de Breteuil, F-92312 Sèvres Cedex France, www.bipm.org > Publications > SI Brochure. |
| [5] Портал «Большая российская энциклопедия» (сайт bigenc.ru, физика) и “Большая российская энциклопедия, 2004 - 2017” (электронная версия – сайт old.bigenc.ru, физика) | Статьи: Метрология, Измеряемое свойство, Измерение, Шкалы измерений и т. д. Articles: Metrology, Measurable Property, Measurement, Measurement Scales, etc. |
| [6] ГОСТ Р МЭК 60027-3-2016. | ГСИ. Обозначения буквенные, применяемые в электротехнике. Часть 3. Логарифмические о относительные величины и единицы измерений. GSI. Letter designations used in electrical engineering. Part 3. Logarithmic relative values and units of measurement. |
| [7] ГОСТ Р МЭК 80000-13-2016 | ГСИ. Величины и единицы. Часть 13. Информатика и информационные технологии. GSI. Values and units. Part 13. Computer science and information technology. |
| [8] ГОСТ Р 8.699-2010 | ГСИ. Величины, единицы, шкалы измерений, используемые в глобальной навигационной спутниковой системе. GSI. Values, units, measurement scales used in the global navigation satellite system. |
| [9] ГОСТ Р 8.739-2011 | ГСИ. Эталоны для координатно-временных измерений. Основные положения. Способы выражения погрешностей. GSI. Standards for coordinate-time measurements. The main provisions. Ways of expressing errors. |
| [10] Дойников А. С. | Лекции по метрологии. – Менделеево: ФГУП «ВНИИФТРИ». – 2021 – 2-е издание, переработанное и дополненное. (Приложение1: Дойников А.С., Крупин Б. Н., Юров Л. В. Метрологическая прослеживаемость координатно-временных измерений. Измерительная техника, 2013, № 9, с. 8-14; Lectures on metrology. – Mendeleevo: FSUE "VNIIFTRI". – 2021 – 2- e edition, revised and expanded. ((Appendix 1: Doynikov A. S., Krupin B. N., Yurov L. V. Metrological Traceability of Coordinate-Time Measurements. Measurement Techniques, Volume 56, issue 9, Published: 12 December 2013, Pages 957 – 964). |
| [11] ISO¦/IEC Guide 98-3-2008 ГОСТ 34100.3-2017 | Guide to the expression of uncertainty in measurement. Межгосударственный стандарт. Неопределённость измерения. Часть 3. Руководство по выражению неопределенности измерения |
| [12] Publication CIE 15: 2004 Публикация МКО 15:2004 | Colorimetry Колориметрия |
| [13] Publication CIE 41: 1978 Публикация МКО 41:1978 | Light as a true visual quantity: Principles of measuremеnt Свет как соответствующая визуальная величина: Принципы измерений |
| [14] JPOTS (UNESCO, 1978). ГСССД 77-84 | Practical salinity scale 1978. Морская вода. Шкала практической солёности 1978 г. |
| [15] OIML Р 14 МОЗМ МР 14 | Polarimetric saccharimeters graduated in accordance with the ICUMSA International Sugar Scale. Сахариметры поляриметрические, градуированные в соответствии с сахарной шкалой Международной комиссии по унификации методов анализа сахара. |

Information data

1. Coordinating organization: «ALL-RUSSIAN SCIENTIFIC RESEARCH

INSTITUTE FOR PHYSICAL-TECHNICAL AND RADIOTECHNICAL

MEASUREMENTS».

Developer of the updated version: A.S.Doynikov

2. COOMET theme 868/RU/23.

3. Document was approved in …th meeting of COOMET Committee.

4. Information about application of the document in organizations – members of

COOMET.

Recommendation is developed on the base of Recommendation COOMET R/GM/ 20:2009 and РМГ 83-2007 «Шкалы измерений. Термины и определения», taking into account the publications that have appeared, presented in the Bibliography, etc.

Recommended system of concepts and terms is intended for use of measurement scales of various types in legal and applied

metrology.