|  | COOMET Recommendation | CОOMET R/GM/21:2024 |
| --- | --- | --- |
|  | Use of concepts “error of measurement” and “uncertainty of measurements”. General principles |  |
| Approved in 21st the meeting of COOMET Committee (27–28 April 2011, Yerevan, Armenia) Clarified and supplemented: at the __ meeting of the COOMET Committee (_______________________) |  |  |

1. SCOPE OF APPLICATION

This recommendation is intended for use in the development of documentation of all types, scientific    and technical, educational and reference literature on metrology used in the performance of work in the field of legislative and applied metrology and standardization.

In  this  document  concepts  «error  of  measurement»  and  «uncertainty  of  measurement»  are  analyzed, and recommendations about logically consistent joint application of these concepts  of various metrological problems are made.  As the basic terminological documents, [1 – 3] are accepted.

Terminological articles in these recommendations, repeating terminological articles of [2], are represented by thin lines.

The previous version of the recommendation (COOMET R/GM/21:2011) was developed on the basis of RMG 91-2009 and updated by updating and supplementing the "Bibliography" [1-11] with new significant documents (publications) and correcting bibliographic references in the text with local notes.

In 	[1] 	basic 	distinction 	of 	concepts 	«error 	of 	measurement» 	and 	«uncertainty 	of  measurements» is underlined, but possibility use of concept «error» is not excluded. Thus, it is  meant that the concrete error always has a certain sign (positive or negative).  Application of  concept «true value of the measured quantity» instead of concept «reference quantity value »  in error definition in [2] it does not change its sense. By definition, unlike «error» concept,  «uncertainty» characterizes dispersion of values which could be attributed to the measured  quantity.

The  incorrectness  of  application  of  concept  «error»  is  indicated  in    its  mixing  with  the  concepts    different  by  implication,  such  as  «  measurement  result  error  characteristics  »,  «  error  confident  limits».  The  error  of  a  certain  result  of  measurement  is  evinced  in  the  considered  experiment  with  a  certain  copy  of  a  measuring  instrument,  and  during  the  evaluation  of    «error  characteristics»  a  set  of  possible  values  of  errors  in  virtual  or  real  experiments  with  various  copies  of  measuring  instruments  of  the  given  type  at  admissible  variation  of  measurement  conditions  is  used.  Therefore the standard evaluation  of  the  standard deviation, non-excluded systematic error and confident limits of a set of errors of

measurement results do not correspond any more to initial definition of an error. Actually,  these  evaluations  characterize  not  an  error,  but  a  dispersion  of  values  attributed  to  the  measured quantity on the basis of the used information, i.e. similar uncertainty.

Concepts «error of measurement» and «uncertainty of measurement» should be applied according to their definitions, without error substitution by evaluation parameter and components of measurement results dispersion.

1/11

2. TERMS AND DEFINITIONS

The following terms with their respective definitions and comments are used in these recommendations:

2.1

measurement error  (error  of  measurement,  error):  measured  quantity  value    minus  a  reference quantity value

N o t e 1: The concept of 'measurement error' can be used both

a) when there is a single reference quantity value to refer to, which occurs if a calibration is made by means of  a measurement  standard  with  a measured  quantity  value having  a  negligible  measurement  uncertainty,  or  if  a  conventional  quantity  value  is  given,  in  which case the measurement error is known, and

b) if a measurand is supposed to be represented by a unique true quantity value or a set of  true quantity values of negligible range, which case the measurement error is not known.

N o t e 2: Measurement error should not be confused with production error or mistake.

[2, article 2.16]

C  o  m  m  e  n  t:  True  value  of  a  quantity  cannot  be  defined.  This concept applies only  in  theoretical researches. In practice, they use reference quantity value Xо (see 4.2), and a error of measurement Δ is defined according to formula:

Δ = Xm – Xо ,

where Xm – the value of quantity received by measurement (result of measurement [2, article

2.9]);

Xо – the value attributed to concrete quantity and accepted, often under the agreement, as a  value, which has an uncertainty acceptable for the given purpose [2, article 2.12].

Thus,  by  definition,  concept  «the  measurement  error»  concerns  only  the  certain  result  of   measurement received with the use of a certain copy of a measuring instrument. The error of   measurement is a certain positive or a negative number. There are no bases to assign to this  concept  a  sense  of  statistical  parameter  of  any  set  of  real  or  guess  values.  «Error  of  measurement» and «uncertainty of measurement»  represent different concepts; they should  not be confused with each other or used in the wrong way [1, point 3.2.2].

2.2.

reference quantity value (reference value): quantity value used as a basis for comparison

with values of quantities of the same kind

N o t e 1: A reference quantity value can be a true quantity value of a measurand, in which  case it is unknown, or a conventional quantity value, in which case it is known.

N o t e 2:  A reference quantity value with associated measurement uncertainty is usually  provided with reference to

a) a material, e.g. certified reference material,

b) a device, e.g. stabilized laser,

c) a reference measurement procedure,

d) a comparison of measurement standards.

[2, article 5.18]

2/11

C o m m e n t:  Concept «conventional quantity value» [2, article 2.12] covers the concept «the  real value of quantity » («the conventional true quantity value») ─ the value of quantity received

experimentally and being so close to the true value, that in the put measuring problem it can be used  instead of that.

2.3. relative error: Ratio Δ/Xo error of  measurement Δ to reference quantity value Xо.

C o m m e n t: It is not recommended to replace in this respect the  reference quantity value by  the  result  of  measurement,  as  it  contradicts  the  concept  of  «  error  of  measurement  »  definition.

2.4.

systematic  measurement  error  (systematic  error  of  measurement,  systematic  error):  component  of  measurement  error  that  in  replicate  measurements  remains  constant  or  varies in a predictable manner

N o t e 1: A reference quantity value for a systematic measurement error is a true quantity  value,  or  a  measured  quantity  value  of  a  measurement  standard  of  negligible  measurement uncertainty, or a conventional quantity value.

N  o  t  e  2:  Systematic  measurement  error,  and  its  causes,  can  be  known  or  unknown.  A  correction can be applied to compensate for a known systematic measurement error.

N  o  t  e  3:    Systematic  measurement  error  equals  measurement  error  minus  random  measurement error.

[2, article 2.17]

C o m m e n t: It is necessary to keep in mind that at definition of a difference of the specified  errors, each of the errors is  taken with the positive or negative sign.

2.5.

random measurement error (random error of measurement, random error): component of measurement error that in repeated measurements varies in an unpredictable manner N o t e 1: A reference quantity value for random measurement error is the average that would ensue from an infinite number of replicate measurements of the same measurand. N o t e 2: Random measurement errors of a set of replicate measurements form a distribution that can be summarized by its expectation, which is generally assumed to be zero, and its variance. N o t e 3: Random measurement error equals measurement error minus systematic measurement error. [2, article 2.19]

C o m m e n t: It is necessary to keep in mind that at definition of a difference of the specified  errors, each of the errors is taken with the positive or negative sign

3/11

2.6.

measurement 	uncertainty 	(uncertainty 	of 	measurement, 	uncertainty): 	Non-negative  parameter characterizing the dispersion of the quantity values attributed to a measurand,  basis on the information used

N o t e 1: Measurement uncertainty includes components arising from systematic effects,  such  as  components  associated  with  corrections  and  the  assigned  quantity  values  of  measurement  standards,  as  well  as  the  definitional  uncertainty.  Sometimes  estimated  systematic  effects  are  not  corrected  for  but,  instead,  associated  measurement  uncertainty  components are incorporated.

N  o  t  e  2:  The  parameter  may  be,  for  example,  a  standard  deviation  called  standard  measurement uncertainty (or a specified multiple of it), or the half-width of an interval,  having a stated coverage probability.

N o t e 3: Measurement uncertainty comprises, in general, many components. Some of these  may be evaluated by Type A evaluation of measurement uncertainty from the statistical  distribution of the quantity values from series of measurements, and can be characterized by  standard deviations. The other components, which may be evaluated by Type B evaluation  of measurement uncertainty, can also be characterized by standard deviations, evaluated  from probability density functions based on experience or other information.

N o t e 4: In general, for a given set of information, it is understood that the measurement  uncertainty  is  associated  with  a  stated  quantity  value  attributed  to  the  measurand.  A  modification of this value results in a modification of the associated uncertainty

[2, article 2.26]

C o m m e n t: The value said in the note 4 is the best estimation of value of the measurand,

and all components of uncertainty, including the components caused by systematic effects, for  example, connected with corrections and standards, lead to dispersion [1, point 2.2.3, Notes 1- 3].

Quantitatively, «uncertainty of measurement» (as a rule) can be characterized by «standard  uncertainty  of  measurement»  –  uncertainty  of  result  of  the  measurement  expressed  as  a  standard  deviation,  or  «the  expanded  measurement  uncertainty»  [1,  points  2.3.1,  2.3.4  and  2.3.5].

Thus, «uncertainty of measurement» as the parameter, characterizes the dispersion of a set of  measurement results possible values in a considered measuring situation, but not an error of  concrete  result  of  measurement.  For  example,  a  case  is  possible,  when  the  result  of  measurement has negligibly small error with large uncertainty [1, point 3.3.1, the note].

4/11

3. RECOMMENDATIONS FOR THE CORRECT APPLICATION OF THE CONCEPTS OF "MEASUREMENT ERROR" AND "MEASUREMENT UNCERTAINTYS"

3.1 Application of concepts «error of measurement» and «uncertainty of measurement»

in concrete metrological situations

3.1.1  Result  of  measurement  is  the  value  of  quantity  received  by  its  measurement  and  uncertainty of measurement . Concrete results of measurements in any metrological situations  unequivocally can and should be characterized by uncertainty. Application of concept error of  measurement result, which is essentially unknown and is specifically indefinable, is possible  only in theoretical reasoning concerning measurement results. Error concept is supposed to be  used  during  measuring  instrument  calibration  and  verification  (see  point  4.1,  note  1,  enumeration a).

3.1.2 Results of experiments, which are carried out during  national  standards  comparisons (key, regional, international) according to the Arrangement [4], are represented  with  the  detailed  data  about  evaluation  of  uncertainty.  Here, the errors characteristics of standards, which are stated in registration certificates for national standards, can be used for evaluation of uncertainty of comparison results.

3.1.3  Calibration  and  measuring  capabilities  of  national  metrological  institutes,  according  to  Appendix  C  of  Agreement  [4],  are  represented  with  indication  expanded  uncertainty of measurement results and coverage factor. Here, it is obligatory to indicate data  about  metrological  traceability  under  transfer  of  appropriate  measurement  scale  or  size  of  measurement unit.

3.1.4 In measurement procedure, operation and calculus complex is described; its fulfillment  provides obtaining of the measurement result with the established indices of accuracy.

Results  of  measurement  according  to  measurement  procedure  are  recommended  to  be  accompanied  by  uncertainty  of  measurement  evaluations.  Measurement  procedure  may  include  information  about  target  uncertainty  of  measurement  [2,  point  2.34].  Standardized  (certified)  measurement  procedure  may  maintain  other  established  indices  of  accuracy  measurements,  for  example  by  [5],  corresponding  of  concrete  setting  and  aria  of  its  application.

3.1.5  At  calibration  of  measuring  instruments,  they  establish,  under  certain  conditions,  the  proportion between values of quantity according to indications of measuring instrument and  appropriate values realized using the standard. According to calibration results, corrections to  measuring instruments indications may be made or values realized by measuring instruments   may be made more exact. As it is specified in Appendix C to Agreement [4], only uncertainty  can  be  a  calibration  accuracy  characteristic.  The  same  concerns  the  results  of  measuring  instruments graduation in the course of calibration.

3.1.6 Measuring instruments metrological characteristics are standardized using the concept  «error»  or  «accuracy  class».  Here,  permissible  error  limits  for  measuring  instruments  of  a  given type are used.

3.1.7  Measuring instruments verification consists in establishing of measuring instrument

(MI) suitability to application on the basis of experimental definition of metrological  characteristics and acknowledgement of their conformity specified requirements. At  verification, measurement standards are used.

MI verification may consist in the following:

a) Determination of MI suitability to application with rejection of those MI, whose error  exceeds limits of the assumed error, established for MI of a given type;

5/11

b) Establishing of conventional true values or gauging characteristics of the MI brought for  verification (including the way of corrections application);

c) Determination of MI suitability to application according to norms of their stability (with  rejection of those MI, whose change of the conventional true value or gauging characteristics  exceeded permissible instability limit established for MI of a given type for inter-verification  interval), and establishing of the conventional true values or gauging characteristics of the  other MIs.

At  verification,  norms  of  measuring  instruments  error  limits  are  operated.  Therefore,  in  verification  it is assumed to specify, in what proportion should be the expanded uncertainty  of   procedures   verification  and  limits  of  errors  of  measuring  instruments  of  the  given confirmed type, and also to specify criteria of measuring instruments validity taking  into  account  uncertainty  of  results  at  verification  (see  Appendix  A).  Here,  general  principles  taking  into  account    uncertainty  of  measurements  in  procedure    evaluation  of  conformity with specified requirements may be used  [6, 9-11].

3.1.8 In construction of a verification chain, accuracy of unit size transfer method may be  characterized by uncertainty of measurement.

3.2 General recommendation.

On the basis of the considered metrological situations, it is possible to assume the general  rule:  in  the  most  metrological  situations,  measurement  results  are  characterized  by  uncertainty, and accuracy of measuring instruments are characterized by error limits. Concept  «error»  is  used  at  comparison  with    reference  quantity  value,  and  evaluation  errors  are  obtained at calibration or verification of measuring instruments.  Thus, concepts "uncertainty"  and "error" are recommended to be used harmoniously, without mutual contraposition and  exclusion of one of them.

6/11

Appendix A  
(reference)

Explanations to the use of concept «uncertainty of measurements» during verification

In the international document OIML D 8 [7], the concept «uncertainty of measurement during  verification of measuring instruments» is used without this provision concretization. Different  versions of its realization are possible due to the diversity of real situations in correlation of  error limits of verified measuring instruments and standards used for that, in correlation of  type  A  and  type  B  evaluated  uncertainties.  For  example,  if  the  expanded  uncertainty  of  measurements  at  verification  (coverage  factor  2)  does  not  exceed  1/3  of  limits  of  error,  uncertainty is ignored. The other possible versions of measuring instruments (MI) suitability  criteria on verification results are:  MI evaluated error doesn’t exceed difference limit of error  and  expanded  measurements  uncertainty  during  verification  (see,  for  example    [8]);  MI  evaluated  error  does  not  exceed  square  root  from  difference  of  squares  limit  of  error  and  expanded  measurements  uncertainty  during  verification.  Such  criteria  of  suitability  may  be  used, in particular, during verification of MI representing the measures, for which the error is  «difference between nominal values of measures and real values of quantities reproduced by  them».  The  sense  of  uncertainty  account  during  verification  is  explained  by  the  graph  in  Figure А.1. The versions represented in this Appendix explain only the principles of concept  «error» and «uncertainty» use, but they are not preferably recommended and don’t cover all  the diversity of possible situations.

U 	U

Xд-Δн 	Xд+ Δв

Xи 		Xд 	X  Xи-U 	Xи+U

Хд – reference (real) value (of standard);

Хи –indication of tested measuring instrument;

н, в  – upper and lower limits of error, according to normative document for measuring  instruments to be tested (usually  н = в = );

U – expanded uncertainty.

Figure А.1 – Diagram of uncertainty taking into account at confirmation of conformity of  tested measuring instrument to limits of error.

The options given in this appendix only explain the principles of using the concepts of "error" and "uncertainty", but are not primarily recommended and do not cover the full variety of possible situations. The role of measurement uncertainty in making decisions on conformity assessment is described in detail in international guidelines and standards

[9-11].

7/11

The updated recommendation RMG 91-2019 [3] contains additional Appendices: Appendix A (reference) "Classification of concepts and terms related to the keywords "error" and "uncertainty"; Appendix B (reference) "Explanations for the use of the concept of "measurement uncertainty" during verification".

The classification of concepts in Appendix A contains detailed comments on their application options, and in Appendix B the explanations are supplemented with specific recommendations for determining the width of the "protective strip" when confirming the conformity of the verified measuring instrument.

8/11

Bibliography

[1] ISO/IEC Guide 98-3:2008, Uncertainty of measurement – Part 3: Guide to the expression of uncertainty in measurement (GUM:1995).

[2] JCGM 200:2008 (E/F). International vocabulary of metrology – Basic and general concepts and associated terms (VIM).

[3] РМГ 91-2019. Государственная система обеспечения единства измерений.  Использование понятий «погрешность измерения» и «неопределённость измерений». Общие принципы.

(State system for ensuring the uniformity of measurements. Use of concepts «error of measurement» and «uncertainty of measurements». General principles).

[4]    Mutual recognition of national measurement standards and of calibration and measurement  certificates issued by national metrology institutes , Paris, 14 October 1999.

[5] ISO 5725-1:1994. Accuracy (trueness and precision) of measurement methods and results. Part 1: General principles and definitions.

[6] ISO 10576-1:2003. Statistical methods. Guidelines for the evaluation of conformity with specified requirements. Part 1. General principles.

[7] International Document OIML D 8, Edition 2004. Measurement standards. Choice, recognition, use, conservation and documentation.

[8] OIML R 111-1-2004. Weights of classes E1, E2, F1, F2, M1, M1-2, М2, М2-3, M3. Part 1: Metrological and technical requirements.

[9] JCGM 106:2012. Evaluation of measurement data - The role of measurement uncertainty in conformity assessment.

[10] Guide OIML G19:2017. The role of measurement uncertainty in conformity assessment decisions in legal metrology.

[11] CISPR 16-4-2:2003, MOD. Electromagnetic compatibility of technical equipment. Specification for radio disturbance and immunity measuring apparatus and methods. Part 4-2. Uncertainties, statistics and limit modelling. Measurement instrumentation uncertainty.

9/11

Information data

1. Coordinating organization: «NATIONAL SCIENTIFIC RESEARCH INSTITUTE FOR

PHYSICOTECHNICAL AND RADIO ENGINEERING MEASUREMENTS».

Developer of the updated version: A.S.Doynikov

2. COOMET theme  869/RU/23.

3. Document was approved at the  __th meeting of COOMET Committee.

4. Information about application of the document in organizations – members of COOMET.

The updated version of the Recommendation was developed taking into account RMG 91-2019 "State system for ensuring the uniformity of measurements. The use of the concepts of "measurement error" and "measurement uncertainty". General Principles", International Dictionary of Metrology. Basic and general concepts and relevant terms (VIM 3), international documents, materials of publications: Doynikov A. S. Lectures on metrology, 2nd edition, Mendeleevo: FSUE "VNIIFTRI". 2021, 376 p., etc.

Recommended conceptually-terminological system and general principles are intended for

use in legislative and applied metrology.

10/11

Contents

1. Scope of application . .  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1                          2. Terms and definitions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2        3. Recommendations concerning correct application of concepts

«a measurement error» and «uncertainty of measurement»    . . . . . . . . . . .            5  Appendix A (reference). Explanations to the use of concept «uncertainty

of measurement» during verification . . . . . . . . . . . . . . . . . . . . . . . . . . . .           7  Bibliography. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .     9  Information data       . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    10

11/11